Supplementary Materials for

Full Title: Non-Adiabatic Reaction Dynamics to Silicon Monosulfide (SiS) - A Key Molecular Building Block to Sulfur-Rich Interstellar Grains

Short Title: Gas Phase Synthesis of Silicon Monosulfide

Srinivas Doddipatla1, Chao He1, Shane Goettl1, Ralf I. Kaiser1*, Breno R. L. Galvão2*, Tom J. Millar3*

1Department of Chemistry, University of Hawai‘i at Mānoa, Honolulu, HI 96822, USA.

2Centro Federal de Educação Tecnológica de Minas Gerais, Av. Amazonas 5253, (30421-169) Belo Horizonte, Brazil.

3School of Mathematics and Physics, Queen’s University Belfast, Belfast BT7 1NN, Northern Ireland, United Kingdom.

*Correspondence to: ralfk@hawaii.edu, brenogalvao@gmail.com, Tom.Millar@qub.ac.uk

This PDF file includes:

Experimental Section

Figure S1 to S3

Table S1 to S5

Experimental Section. The crossed molecular beam experiments of ground state silicon atoms (Si; 3P) with deuterium sulfide (D2S; X1A1) were performed in gas phase under single collision conditions exploiting a crossed molecular beam machine (65). Note that the reaction was conducted with deuterium sulfide, but not with hydrogen sulfide (≥ 99 %, Sigma-Aldrich) due to the presence of traces of dihydrogendisulfide in the hydrogen sulfide lecture bottle leading to non-reactive scattering signal at the prospective products of 61 amu (HSiS) and 60 amu (SiS). This problem was overcome by using deuterium sulfide. A pulsed supersonic beam of neon-seeded ground state silicon atoms was produced via ablation of a rotating silicon rod at 266 nm (10 ± 1 mJ per pulse; 30 Hz). The ablated silicon atoms were entered into neon carrier gas (Ne; 99.999%; Specialty Gases) released from a pulsed piezoelectric valve operating at 60 Hz, a pulse width 80 µs, and a peak voltage -400 V. A four-slit chopper wheel rotating at 120 Hz selected a part of the pulsed beam defined by a peak velocity (vp) and speed ratio (S) of 984 ± 15 m s-1 and 5.9 ± 0.8, respectively. Laser induced fluorescence revealed that silicon atoms are in their electronic ground state (3P) (77). This section of the beam intersected perpendicularly an early section of a pulsed beam of deuterium sulfide (97 % D; Sigma-Aldrich) characterized by a peak velocity (vp) of 801 ± 21 ms-1 and a speed ratio (S) of 12.8 ± 0.8. The neutral reactive scattering products are ionized by electron impact at energy of 80 eV and 2 mA emission current; the ions are then extracted and mass filtered by a triply differentially pumped quadrupole mass filter operating in the TOF mode and detected by Daly-type ion counter; up to 2 × 105 TOF spectra were accumulated at each angle. The laboratory angular distributions were then obtained by integrating the TOF at each angle and normalized to the intensity of the TOF at the center-of-mass angle (CM). The reaction dynamics were extracted by converting the data from the laboratory frame to the CM reference frame exploiting a forward-convolution routine (45, 46)  providing the CM translational energy P(ET) and angular T(θ) flux distribution for each channel. For barrierless reactions without threshold, a reactive cross-section with an ET-1/3 energy dependence; for reactions with a threshold (E0), a (1-E0/ET) energy dependence was utilized (78) . Branching ratios were calculated using the method of Krajnovich et al.  (79) .

Fig. S1. Potential energy surface for the reactions of hydrogen sulfide (H2S) with atomic silicon (Si) at the MRCI-F12/cc-pVQZ-F12//CASSCF/aug-cc-pV(Q+d)Z level. Relative energies are given in units of kJ mol-1. Plain numbers are zero-point energies corrected for hydrogeneted reactants, whereas numbers in parentheses are for deuterated reactants. Colors of the atoms are defined as sulfur (yellow), silicon (blue), and hydrogen (gray). The cartesian coordinates and vibrational frequencies are compiled in the (Table S3).

Fig. S2. Structures of minima-on-the-seam-of-crossings (MSXs).

Fig. S3. Computed geometries of the exit transition states leading to the formation of SiS (p1) plus molecular deuterium from intermediates cis-S2 (left) and S3 (right).

Table S1.  Peak velocities (vp) and speed ratios (S) of ground state atomic silicon (Si(3P)) and deuterium sulfide (D2S; X1A1) beams along with the collision energy (EC) and center-of-mass angle (ΘCM).

| beam | vp (m s-1) | S | EC (kJ mol-1) | ΘCM (deg) |
| --- | --- | --- | --- | --- |
| Si(3P) | 984 ± 15 | 5.9 ± 0.8 |  |  |
| D2S (X1A1) | 801 ± 21 | 12.8 ± 0.8 | 12.7 ± 0.4 | 46.3 ± 0.8 |

Table S2. D and D2 loss product mass combinations of silicon and sulfur isotopes from the reaction of ground state atomic silicon and deuterium sulfide. Isotope abundance given in the parenthesis.

| Si + D2S |  | D232S (94.93%) 36 | D233S (0.76%) 37 | D234S (4.29%) 38 |
| --- | --- | --- | --- | --- |
| D Loss | 28Si (92.23%) 28 | 28Si32SD 62 | 28Si33SD 63 | 28Si34SD 64 |
|  | 29Si (4.68%) 29 | 29Si32SD 63 | 29Si33SD 64 | 29Si34SD 65 |
|  | 30Si (3.09%) 30 | 30Si32SD 64 | 30Si33SD 65 | 30Si34SD 66 |
| D2 Loss | 28Si (92.23%) 28 | 28Si32S 60 | 28Si33S 61 | 28Si34S 62 |
|  | 29Si (4.68%) 29 | 29Si32S 61 | 29Si33S 62 | 29Si34S 63 |
|  | 30Si (3.09%) 30 | 30Si32S 62 | 30Si33S 63 | 30Si34S 64 |

Table S3. Optimized Cartesian Coordinates (Å) and Vibrational Frequencies (cm-1) of the reactants, products, transition states, and MSX on the singlet and triplet potential energy surfaces.

| Species | Vibrational Frequencies (cm-1) | Cartesian Coordinates (Å) Atom X Y Z |
| --- | --- | --- |
| H2S | 1198.66 2649.17 2673.60 | S 0.000000 -0.000000 -0.054937 H 0.000000 0.980451 0.873707 H 0.000000 -0.980451 0.873707 |
| T1:H2SSi | 180.99 452.65 456.47 1195.18 2623.98 2642.47 | S -0.000000 -0.057984 1.143524 H 0.983506 0.816017 1.464236 H -0.983516 0.816007 1.464236 Si -0.000000 0.005839 -1.415638 |
| T2: HSSiH-T | 356.19 523.32 629.64 712.77 2324.36 2649.52 | S -0.017379 -0.042315 -1.003641 H -1.202415 0.140549 1.871912 H 0.117356 1.276157 -1.202526 Si 0.058780 -0.002538 1.121647 |
| T3:SSiH2 | 526.65 546.81 779.02 983.76 2315.08 2332.64 | Si 0.000013 -0.415804 1.128159 H 1.215114 0.161026 1.738861 H -1.215074 0.161026 1.738887 S -0.000008 0.093786 -0.946350 |
| TS-diss-T1-HSSi+H | 1241.64i 280.81 468.18 550.99 722.52 2553.70 | S -0.063268 -0.051584 -0.956192 H -0.248702 1.264111 -1.243756 H 1.496326 0.136322 -2.112130 Si 0.027446 0.008625 1.211943 |
| TS-diss-T2-SSiH+H | 1160.07i 293.08 369.05 670.50 696.85 2245.18 | S -0.084482 0.011817 -0.923990 Si 0.033259 -0.058072 1.052597 H -0.057254 1.158677 1.902668 H 1.817687 0.083575 -1.842725 |
| TS-diss-T2-SiS+H2 | 1576.14i 409.61 650.42 858.49 1656.42 1898.54 | S -0.034198 -0.277604 -1.195345 H 0.030896 1.657612 0.666896 H 0.010530 1.293253 -0.246914 Si 0.003231 0.054909 0.946863 |
| TS-iso-T1-T2 | 764.49i 349.98 533.83 790.94 1458.44 2594.07 | S -0.033970 -0.042377 -1.085872 H -0.995963 0.645453 0.120600 H 0.574657 1.150514 -1.292633 Si 0.019234 -0.010937 1.317689 |
| TS-iso-T2-T3 | 1367.75i 561.52 652.40 680.25 1713.17 2295.52 | Si 0.089515 -0.429029 1.096976 H 1.192570 0.168324 1.880465 H -0.798269 0.729392 0.363414 S -0.063646 -0.018946 -0.962976 |
| S1: H2SSi | 237.01 447.66 592.03 1328.08 2170.30 2378.53 | S -0.154062 0.004596 0.971997 H 0.265997 -1.004536 1.797108 H 0.269769 0.997112 1.825832 Si 0.052595 -0.007143 -1.160358 |
| Trans-S2: (HSSiH) | 526.81 604.66 684.12 954.11 2191.62 2669.86 | S 0.075031 0.191007 -1.262024 H -1.532691 -0.016944 0.873414 H 1.338415 0.639985 -1.307538 Si -0.115595 0.490031 0.842247 |
| Cis-S2: (HSSiH) | 502.68 507.86 696.89 863.10 2188.78 2591.10 | S 0.000005 -0.040326 -1.004445 H 0.000139 1.438803 1.347005 H -0.000157 1.284931 -1.229789 Si -0.000006 -0.051717 1.142382 |
| S3: SSiH2 | 676.08 716.44 742.74 1108.26 2369.51 2375.63 | Si -0.000001 -0.000001 0.972899 H 1.196893 0.000017 1.826892 H -1.196846 0.000017 1.826928 S -0.000000 0.000000 -0.967161 |
| TS-diss-S1-SiSH+H | 867.31i 151.61 431.08 553.68 865.07 2539.96 | S 0.007998 -0.037728 0.908483 H 1.178415 0.366882 1.442400 H -0.988090 0.421180 2.484369 Si -0.015961 0.014785 -1.177971 |
| TS-diss-cisS2-SiS+H2 | 1623.18i 595.96 987.12 1012.77 1593.39 1761.39 | S -0.032598 -0.232483 -1.143046 H 0.031563 1.693082 0.660862 H 0.009419 1.255357 -0.273546 Si 0.002075 0.012213 0.927230 |
| TS-isom-S1-cisS2 | 1077.46i 232.48 590.27 1275.32 1929.94 2284.54 | S 0.010733 -0.060461 1.064112 H 0.702898 0.972366 0.386708 H -0.673559 0.984744 1.651843 Si -0.013305 -0.001219 -1.287859 |
| TS-isom-cisS2-transS2 | 665.95i 405.95 625.92 692.29 2346.37 2539.95 | S 0.249499 -0.142744 -1.332041 H -1.422203 0.642659 0.801443 H 0.787729 1.101897 -1.306465 Si -0.051067 0.140840 0.896848 |
| TS-isom-transS2-S3 | 1663.18i 548.54 579.71 786.17 1784.87 2224.48 | Si 0.116033 0.446121 1.102345 H 1.280799 -0.224639 0.256350 H -0.940142 -0.277625 1.871131 S -0.101890 0.027183 -0.934486 |
| TS-S3-SiS+H2 | 1342.25i 512.68 539.05 678.02 1143.88 2305.40 | Si 0.000000 0.103972 0.984152 H 0.000000 -1.614781 1.503071 H 0.000000 -0.507192 2.338307 S 0.000000 -0.024369 -0.982915 |
| SiS (singlet) | 733.92 | S 0.000000 0.000000 -0.908295 Si 0.000000 0.000000 1.036832 |
| SiS (triplet) | 488.72 | S 0.000000 0.000000 -1.021078 Si 0.000000 0.000000 1.165575 |
| SSiH | 690.09 701.86 2199.56 | Si 0.000000 -0.059692 -1.011007 S 0.000000 0.016031 0.944641 H 0.000000 1.153351 -1.875656 |
| HSSi | 502.92 622.25 2559.67 | S 0.003305 0.176352 -1.019021 H -0.029846 -1.129548 -1.347360 Si -0.001829 -0.075434 1.089941 |
| H2 | 4285.17 | H 0.000000 0.000000 -0.377382 H 0.000000 0.000000 0.377382 |
| MSX1 | 2573.91 1440.46 774.33 578.73 283.65 | S -0.005107 -0.049059 -1.084659 H -1.049825 0.654129 0.173867 H 0.596803 1.146190 -1.308290 Si 0.022088 -0.008608 1.278866 |
| MSX2 | 2669.15 1803.68 812.94 660.83 564.82 | S -0.013581 -0.044940 -0.995519 H -1.043264 -0.025168 2.073599 H -0.043435 1.275840 -1.229478 Si 0.054503 0.006416 1.106105 |
| MSX3 | 2130.73 1792.85 1072.02 754.85 277.53 | Si -0.067398 -0.038147 -1.107977 H 0.559253 1.010089 -2.000754 H 1.377742 -0.461322 -0.653008 S -0.001854 0.016165 1.054052 |

Table S4: Source data from Tercero et al(28)

|  | OHC | OPl | O15 |
| --- | --- | --- | --- |
| T(K) | 225 | 125 | 200 |
| n(H2) (cm-3) | 5 ×107 | 106 | 5 ×106 |
| Source diam (arcsec) | 10 | 30 | 10 |
| Source diam (cm) | 6×1016 | 1.8×1017 | 6×1016 |
| N(SiS) (cm-2) | (3.0±0.7)×1014 | (3.5±0.8)×1014 | (7.0±1.7)×1014 |
| N(H2) (cm-2) | 4.2×1023 | 2.1 ×1023 | 1023 |
| f(SiS) | (7.1±1.7)×10-10 | (1.7±0.4)×10-9 | (7.0±1.7)×10-9 |

Fractional abundances are difficult to determine for such small, dense sources because of approximatons made about optical depth of observed emission lines, source size, density, and excitation temperature. As an example, the Orion Plateau source was also observed in SiS by Ziurys(80) in the 5-4 and 6-5 transitions with a relatively large beam, 58 and 49 arcsec, larger than the source size, 30 arcsec.  In this paper she derived N(SiS) = (2.8-3.9)×1015 cm-2 and adopted N(H2) = 1024 cm-2 (although she noted that its value could be anywhere form (5-50)×1023 cm-2) to find f(SiS) = (2.8-3.9)×10-9. In this analysis she adopted a rotational temperature of 230K, higher than that currently thought (125K). Ziurys (81)reobserved these two transitions with a different telescope and added the 15-14 transition to them.  In this paper, she used a rotational temperature of 100K and derived N(SiS) = 4×1015 cm-2 for a 10 arcsec source and 1.4×1014 cm-2 for a 27 arcsec source, closer to the actual size of the plateau.

Table S5: Initial fractional abundances, f(X), of species relative to H2

| Species | Abundance | Species | Abundance |
| --- | --- | --- | --- |
| He | 1 ×10-1 | CH3OH | 1 ×10-6 |
| H2O | 2 ×10-4 | N2 | 4 ×10-7 |
| CO | 1 ×10-4 | SiH4 | 2 ×10-7 |
| NH3 | 2 ×10-5 | OCS | 1 ×10-7 |
| H2S | 2 ×10-6 | CH4 | 1 ×10-7 |